Test

Matlab code (avi):

%% Project 1 - Matlab code (Use pines A0 and D9)

%% --- SETUP ---

board = 'Uno';                 % Arduino board (e.g., 'Uno', 'Mega2560')

port  = [];                    % [] = auto-detect; else 'COM5' or '/dev/tty.usbmodem...'

pot   = 'A0';                  % Potentiometer wiper to analog pin A0

pin   = 'D9';                  % Servo control signal pin

Vref  = 5;                     % Reference voltage for readVoltage()

Vth   = 2.5;                   % Threshold voltage to trigger servo ON

onDeg = 90;                    % Servo angle when ON (degrees)

offDeg= 0;                     % Servo angle when OFF (degrees)

Ts    = 0.02;                  % Loop delay (s)

T_read= 12;                    % Duration for Test 1 (s)

T_thr = 12;                    % Duration for Test 3 (s)

if isempty(port)

a = arduino('Libraries','Servo');                 % Auto-detect board/port

else

a = arduino(port, board, 'Libraries','Servo');    % Explicit board/port

end

s = servo(a, pin, 'MinPulseDuration',1e-3, 'MaxPulseDuration',2e-3);  % Attach servo on D9

toPos = @(deg) max(0, min(1, deg/180));              % Map 0–180° to 0–1 position

%% --- QUICK CHECK ---

writePosition(s, toPos(offDeg)); pause(0.3);          % Go to OFF

writePosition(s, toPos(onDeg));  pause(0.3);          % Go to ON

writePosition(s, toPos(offDeg));                      % Return to OFF

%% --- TEST 1: READ & PLOT POT ---

f = figure('Name','A0 Voltage');                      % Live plot window

h = animatedline('LineWidth',1.5); grid on;           % Create animated line

xlabel('Time (s)'); ylabel('Voltage (V)'); ylim([0 Vref]);

t0 = tic;                                             % Start timer

while ishandle(f) && toc(t0) < T_read

v = readVoltage(a, pot);                          % Read 0..Vref from A0

addpoints(h, toc(t0), v);                         % Append (t, v)

drawnow limitrate;                                % Update plot efficiently

pause(Ts);                                        % Control loop rate

end

%% --- TEST 2: COMMAND SERVO (ON/OFF) ---

writePosition(s, toPos(offDeg)); pause(0.5);          % OFF

writePosition(s, toPos(onDeg));  pause(1.0);          % ON

writePosition(s, toPos(offDeg)); pause(0.5);          % OFF

%% --- TEST 3: THRESHOLD CONTROL ---

t0 = tic;                                             % Reset timer

while toc(t0) < T_thr

v = readVoltage(a, pot);                          % Read pot voltage

if v >= Vth                                      % Compare to threshold

writePosition(s, toPos(onDeg));               % Set ON angle

else

writePosition(s, toPos(offDeg));              % Set OFF angle

end

pause(Ts);                                        % Control loop rate

end

%% --- END ---

writePosition(s, toPos(offDeg));                      % Park at OFF