Camera Subsystem

| Customer Req. | Engineering Requirements |
| --- | --- |
| The system must detect all pins and or colored stickers placed in the camera view | The camera and image processing in MATLAB must correctly identify 100% of the pins and or shapes in the field of view under standard lighting conditions |
| The system must identify the position of each pin and sticker | The centroid of each object must be detected within +-5 pixels of its true location in the camera frame |
| The system must be able to differentiate between multiple objects of varying shapes and colors | The vision algorithm must correctly classify at least 95% of all test cases involving multiple different shapes at different angles and rotations |
| The system must accurately determine the color of each sticker | Color classification accuracy must be at least 95% for all defined colors: red, green, yellow, and blue |
| The sensing process must be stable and repeatable | Repeated detection of a stationary configuration must produce consistent position and color results with less than 5% variance between trials. |
| The system must provide real-time feedback to the GUI | The full image capture and processing cycle must complete in less than a second per frame |
| The sensing system must be low-cost and easy to integrate | The selected camera must be below $30 and interface directly with the Image Acquisition toolbox in MATLAB via USB connection |