close all;

clc;

clear;

% Dialog box prompt for imaging parts

function dialogBox(currentImage)

d = dialog('Position',[300 300 250 150],'Name','Background Acquisition');

textString = 'Click advance when ' + currentImage + ' is ready for imaging';

txt = uicontrol('Parent',d,...

'Style','text',...

'Position',[20 80 210 40],...

'String',textString); %#ok<NASGU>

btn = uicontrol('Parent',d,...

'Position',[85 20 70 25],...

'String','Advance',...

'Callback','delete(gcf)'); %#ok<NASGU>

uiwait(d);

end

%% Function to send angle to Simulink via base workspace

function sendAngleToMotor(angle)

% Send angle to Simulink via base workspace

assignin('base', 'targetAngle', angle);

% Set a flag to indicate new angle is available

assignin('base', 'newAngleAvailable', 1);

set_param('working', 'SimulationCommand', 'update')

% Display confirmation

fprintf('Angle %.2f° sent to Simulink workspace\n', angle);

% Optional: If you want to automatically start/update Simulink model

startSimulinkModel(); % Uncomment if needed

end

%% Optional: Function to automatically control Simulink model

function startSimulinkModel()

modelName = 'working';

% Check if model is loaded

if ~bdIsLoaded(modelName)

try

load_system(modelName);

fprintf('Simulink model "%s" loaded\n', modelName);

catch

fprintf('Could not load Simulink model "%s". Please check the model name.\n', modelName);

return;

end

end

% Start simulation (optional - uncomment if you want auto-start)

% set_param(modelName, 'SimulationCommand', 'start');

% fprintf('Simulation started\n');

end

%% Function for moving motor to selected pin

function angle = moveMotorToPin(objectLoc, fig, backGnd, motorPosition)

fprintf("Calculating movement angle...\n");

% Get image dimensions

imgHeight = size(backGnd, 1);

imgWidth = size(backGnd, 2);

% Motor is at the bowler's position: far left middle of image

% 0° = straight right (east), positive = counterclockwise (up), negative = clockwise (down)

% Define motor position (bowler's position)

motorX = motorPosition.x;  % Far left (small X value)

motorY = motorPosition.y;  % Middle of image height

% Pin position in image coordinates

pinX = objectLoc(1);

pinY = objectLoc(2);

% Calculate vector from motor to pin

dx = pinX - motorX;  % Horizontal distance (positive = pin is to the right of motor)

dy = motorY - pinY;  % Vertical distance (positive = pin is above motor)

% Calculate angle using atan2

% atan2(dy, dx) gives angle relative to positive X-axis (east)

% This matches: 0° = east, 90° = north, -90° = south

angle = rad2deg(atan2(dy, dx));

fprintf("Motor position: (%.1f, %.1f)\n", motorX, motorY);

fprintf("Pin position: (%.1f, %.1f)\n", pinX, pinY);

fprintf("Vector to pin: dx=%.1f, dy=%.1f\n", dx, dy);

fprintf("Calculated angle: %.2f degrees\n", angle);

close(fig);

% Send the angle to your motor controller via Simulink

sendAngleToMotor(angle);

pause(10);

sendAngleToMotor(0);

return;

end

%% Initialize camera

cam = webcam('Brio 100');

% Initialize variables in base workspace for Simulink

evalin('base', 'targetAngle = 0;'); % Initial angle

evalin('base', 'newAngleAvailable = 0;'); % Flag to indicate new angle

fprintf('Initialized workspace variables for Simulink:\n');

fprintf('  targetAngle = 0\n');

fprintf('  newAngleAvailable = 0\n');

%% Background image capture

fprintf("Waiting to capture background image...\n");

dialogBox("background image")

rawBackGnd = snapshot(cam);

gameState.backGnd = rot90(rawBackGnd, 2);

%% Game image capture

fprintf("Waiting to capture game state image...\n")

dialogBox("game board")

rawForeGnd = snapshot(cam);

gameState.foreGnd = rot90(rawForeGnd, 2);

[height,width,~] = size(gameState.foreGnd);

%% Motor position (bowler's position - far left middle)

motorPosition.x = width * 0.2;  % 20% from left edge (adjust as needed)

motorPosition.y = height * 0.5; % 50% from top (middle vertically)

%% Subtract background and game state

gameState.backGndSUBTRACTED = rawBackGnd - rawForeGnd;

[rawHeight, rawWidth, ~] = size(rawBackGnd);

%% Fixing image

for i=1:rawHeight

for j=1:rawWidth

if (gameState.backGndSUBTRACTED(i,j,1) > 2) || ...

(gameState.backGndSUBTRACTED(i,j,2) > 2) || ...

(gameState.backGndSUBTRACTED(i,j,3) > 2)

gameState.backGndSUBTRACTED(i,j,:) = [175,200,175];

end

end

end

figure();

imshow(gameState.backGndSUBTRACTED);

gameState.backGndSUBTRACTED = rgb2gray(gameState.backGndSUBTRACTED);

%% Get foreground binary

gameState.foreGndBINARY = imbinarize(gameState.backGndSUBTRACTED);

%% Erode and dilate to get rid of noise

SE = strel('disk',20);

gameState.foreGndBINARY = imerode(gameState.foreGndBINARY, SE);

gameState.foreGndBINARY = imdilate(gameState.foreGndBINARY, SE);

figure();

imshow(gameState.foreGndBINARY);

%% Get region information

STATS = regionprops(gameState.foreGndBINARY, 'all');

%% Acquire Pin Locations and transform coordinates for rotated display

pinPos = [];

for i = 1 : length(STATS)

if STATS(i).Area > 200 && STATS(i).Area < 100000

centroid_x = STATS(i).Centroid(1);

centroid_y = STATS(i).Centroid(2);

% Transform coordinates for 180-degree rotated display

rotated_x = rawWidth - centroid_x;

rotated_y = rawHeight - centroid_y;

pinPos = [pinPos; rotated_x, rotated_y];

end

end

%% GUI Configure

screenSize = get(0, 'ScreenSize');

screenWidth = screenSize(3);

screenHeight = screenSize(4);

scaleFactor = 0.8;

maxFigHeight = screenHeight * scaleFactor;

maxFigWidth = screenWidth * scaleFactor;

if height > maxFigHeight || width > maxFigWidth

heightRatio = maxFigHeight / height;

widthRatio = maxFigWidth / width;

scale = min(heightRatio, widthRatio);

figWidth = width * scale;

figHeight = height * scale;

else

figWidth = width;

figHeight = height;

end

figX = (screenWidth - figWidth) / 2;

figY = (screenHeight - figHeight) / 2;

fig = uifigure('Name', 'Robot Bowling Control', ...

'Position', [figX, figY, figWidth, figHeight]);

ax = uiaxes(fig, 'Position', [0, 0, figWidth, figHeight]);

imshow(gameState.foreGnd, 'Parent', ax);

title(ax, 'Select a Pin to Target');

% Draw motor position on the image for visualization

hold(ax, 'on');

plot(ax, motorPosition.x, motorPosition.y, 'ro', 'MarkerSize', 10, 'LineWidth', 2);

hold(ax, 'off');

ax.DataAspectRatio = [1 1 1];

ax.XLim = [1 width];

ax.YLim = [1 height];

xScale = figWidth / width;

yScale = figHeight / height;

% Overlay buttons only for visible pins

for i = 1:size(pinPos, 1)

scaledX = pinPos(i,1) * xScale;

scaledY = pinPos(i,2) * yScale;

buttonY = figHeight - scaledY - 15;

btn = uibutton(fig, 'push', ...

'Text', sprintf('Select'), ...

'Position', [scaledX-15, buttonY, 30, 30], ...

'ButtonPushedFcn', @(btn, event) moveMotorToPin([pinPos(i,1), pinPos(i,2)], ...

fig, gameState.backGnd, motorPosition), ...

'FontSize', 6);

btn.BackgroundColor = [0.2 0.8 0.2];

end