NOTE: CHANGE RED HILIGHTED AREA TO CALIBRATION VALUES FROM HSV THRESHOLD CODE

close all;

clc;

clear;

%% -------------------- CONFIG FLAGS -------------------- %%

USE_LIVE_CAMERA      = true;   % true = use webcam, false = load an image from file

USE_DEFAULT_WEBCAM   = false;   % true = MacBook built-in (default cam), false = named cam below

CAMERA_NAME          = 'Brio 100';   % used if USE_DEFAULT_WEBCAM = false

TEST_IMAGE_PATH      = 'testFrame.png';  % used if USE_LIVE_CAMERA = false

desiredMaxPins       = 6;      % max number of pins to detect

%% -------------------- Lane & Gear Parameters -------------------- %%

% Lane geometry

laneWidth_in = 6;              % total lane width in inches

halfLane_in  = laneWidth_in/2; % 3 in each side

% Gear / rack parameters (Mod 1, 16-tooth spur gear)

module_mm   = 1;      % metric module (mm per tooth)

gearTeeth   = 16;

mm_per_in   = 25.4;

pitchDiameter_mm = module_mm * gearTeeth;       % d = m*z

travelPerRev_mm  = pi * pitchDiameter_mm;       % linear travel per revolution

travelPerRev_in  = travelPerRev_mm / mm_per_in; % in/rev

maxRotations = halfLane_in / travelPerRev_in;   % max rev from center to side

fprintf('Travel per rev: %.3f in\n', travelPerRev_in);

fprintf('Max rotations from center: %.3f rev\n', maxRotations);

%% -------------------- Fixed lane pixel measurements -------------------- %%

% NOTE: these should come from the calibration.

% Using same values here, but adding a safety check to avoid divide-by-zero.

laneLeft_px_meas  = 900.0;   % one side of lane

laneRight_px_meas = 900.0;   % other side of lane

laneLeft_px  = min(laneLeft_px_meas,  laneRight_px_meas);

laneRight_px = max(laneLeft_px_meas,  laneRight_px_meas);

laneCenter_px = (laneLeft_px + laneRight_px) / 2;

pixelSpan     = abs(laneRight_px - laneLeft_px);

if pixelSpan == 0

warning('laneLeft_px_meas == laneRight_px_meas; inchesPerPixel undefined. Set proper calibration values.');

inchesPerPixel = NaN;

else

inchesPerPixel = laneWidth_in / pixelSpan;

end

fprintf('Lane left pixel (stored):   %.1f\n', laneLeft_px);

fprintf('Lane right pixel (stored):  %.1f\n', laneRight_px);

fprintf('Lane center pixel (stored): %.1f\n', laneCenter_px);

fprintf('Pixel span:                 %.3f px\n', pixelSpan);

fprintf('Inches per pixel (fixed):   %.5f in/px\n', inchesPerPixel);

%% expose to base for functions using evalin

assignin('base', 'laneLeft_px', laneLeft_px);

assignin('base', 'laneRight_px', laneRight_px);

assignin('base', 'laneCenter_px', laneCenter_px);

assignin('base', 'inchesPerPixel', inchesPerPixel);

%% -------------------- Initialize Camera & Simulink Vars -------------------- %%

% Only need this if you are using the live camera

if USE_LIVE_CAMERA

if USE_DEFAULT_WEBCAM

% Default camera (MacBook webcam, etc.)

cam = webcam;

else

% Try named camera (e.g., Brio 100)

try

cam = webcam(CAMERA_NAME);

catch ME

warning('Could not open camera "%s": %s. Falling back to default webcam.', ...

CAMERA_NAME, ME.message);

cam = webcam;

end

end

% Optional: tame exposure a bit (adjust as needed / may not work on Mac)

try

cam.ExposureMode = 'manual';

cam.Exposure     = -5;

catch

disp('Could not set manual exposure on this camera (OK to ignore).');

end

end

% Variables used by Simulink (in base workspace) – safe even if nothing connected

evalin('base', 'targetRot = 0;');

evalin('base', 'newTargetRotAvailable = 0;');

evalin('base', 'currentRot = 0;');

assignin('base', 'travelPerRev_in', travelPerRev_in);

assignin('base', 'maxRotations', maxRotations);

assignin('base', 'halfLane_in', halfLane_in);

fprintf('Initialized workspace variables for Simulink.\n');

%% -------------------- Capture Game Image -------------------- %%

fprintf("Waiting to capture game state image...\n");

dialogBox("game board");

if USE_LIVE_CAMERA

rawImg = snapshot(cam);

else

rawImg = imread(TEST_IMAGE_PATH);

end

% OPTIONAL HORIZONTAL FLIP:

fore = fliplr(rawImg);  % turn off if you don't want mirroring

gameState.foreGnd = fore;

[height,width,~] = size(gameState.foreGnd);

figure;

imshow(gameState.foreGnd);

title('Raw Game Image (possibly flipped)');

%% -------------------- Pin Detection via HSV + shape filtering ----------- %%

hsvImg = rgb2hsv(gameState.foreGnd);

h = hsvImg(:,:,1);

s = hsvImg(:,:,2);

v = hsvImg(:,:,3);

% --- 1) Color & brightness thresholds (tune for your pins) ---

% Orange-ish hue band (approx): adjust if needed

hsvImg = rgb2hsv(gameState.foreGnd);

h = hsvImg(:,:,1);

s = hsvImg(:,:,2);

v = hsvImg(:,:,3);

% === Tight thresholds learned from your ROI ===

% Original stats:

%   h in [0.037, 0.060], s > 0.690, v in [0.867, 1.000]

% Add a little margin so it's not ultra brittle:

hLow  = 0.037 - 0.010;   % 0.027

hHigh = 0.060 + 0.010;   % 0.070

sLow  = 0.69  - 0.05;    % 0.64

vLow  = 0.867 - 0.05;    % ~0.82

vHigh = 1.00;            % keep the top at 1.0

maskHue = (h >= hLow  & h <= hHigh);

maskSat = (s >= sLow);

maskVal = (v >= vLow  & v <= vHigh);

pinMask = maskHue & maskSat & maskVal;

figure;

imshow(gameState.foreGnd);

hold on;

hMask = imshow(cat(3, ones(size(pinMask)), zeros(size(pinMask)), zeros(size(pinMask))));

set(hMask, 'AlphaData', 0.3 * pinMask);   % red tint only where mask = 1

title('Mask overlay (red = pinMask)');

hold off;

% --- 2) Morphological cleanup ---

pinMask = bwareaopen(pinMask, 80);     % drop tiny specks

SE = strel('disk', 3);

pinMask = imopen(pinMask, SE);         % smooth small noise

pinMask = imclose(pinMask, SE);        % close gaps

pinMask = imfill(pinMask, 'holes');    % solid blobs

figure;

imshow(pinMask);

title('Cleaned Binary Mask (Color-thresholded pins)');

% --- 3) Shape filtering: keep round blobs & max 6 pins ---

cc    = bwconncomp(pinMask);

stats = regionprops(cc, 'Area','Centroid','Perimeter');

fprintf('Found %d candidate blobs before filtering.\n', numel(stats));

pinPos = [];

if isempty(stats)

warning('No regions found – adjust HSV thresholds (hLow/hHigh, sLow, vLow).');

else

areas = [stats.Area];

perims = [stats.Perimeter];

% Circularity measure: 1 = perfect circle

circularity = 4*pi*areas ./ (perims.^2 + eps);

minArea          = 100;   % adjust if your pins are larger/smaller in pixels

circularityMin   = 0.6;   % 0.6–0.8 is a good starting range

keepMask = (areas >= minArea) & (circularity >= circularityMin);

statsKeep = stats(keepMask);

fprintf('Keeping %d blobs after area & circularity filtering.\n', numel(statsKeep));

if isempty(statsKeep)

warning('No blobs passed shape filtering – try loosening thresholds.');

else

areasKeep = [statsKeep.Area];

[~, idxSort] = sort(areasKeep, 'descend');

keepN = min(desiredMaxPins, numel(idxSort));

selected = statsKeep(idxSort(1:keepN));

pinPos = zeros(keepN, 2);

for k = 1:keepN

pinPos(k,:) = selected(k).Centroid;

end

end

end

fprintf('Using %d pin centroid(s).\n', size(pinPos,1));

% Debug overlay to check centroids actually used

figure;

imshow(gameState.foreGnd);

title('Debug: detected pin centroids overlay');

hold on;

for i = 1:size(pinPos,1)

plot(pinPos(i,1), pinPos(i,2), 'r+', 'MarkerSize', 15, 'LineWidth', 2);

end

hold off;

%% -------------------- GUI Configuration (smaller window) -------------------- %%

scale = 0.5;   % 0.5 = 50% of image size

figWidth  = width  * scale;

figHeight = height * scale;

screenSize   = get(0, 'ScreenSize');

screenWidth  = screenSize(3);

screenHeight = screenSize(4);

figX = (screenWidth  - figWidth ) / 2;

figY = (screenHeight - figHeight) / 2;

fig = uifigure('Name', 'Robot Bowling Control', ...

'Position', [figX, figY, figWidth, figHeight]);

ax = uiaxes(fig, 'Position', [0, 0, figWidth, figHeight]);

imshow(gameState.foreGnd, 'Parent', ax);

title(ax, 'Select a Pin to Target');

% Motor (bowler) position in IMAGE coordinates (right side middle)

motorPosition.x = width * 0.92;

motorPosition.y = height * 0.50;

hold(ax, 'on');

plot(ax, motorPosition.x, motorPosition.y, 'ro', 'MarkerSize', 10, 'LineWidth', 2);

hold(ax, 'off');

ax.DataAspectRatio = [1 1 1];

ax.XLim = [1 width];

ax.YLim = [1 height];

% Scale factors for image → GUI button coordinates

xScale = figWidth  / width;

yScale = figHeight / height;

% Overlay buttons at pin positions

for i = 1:size(pinPos, 1)

uix = pinPos(i,1) * xScale;

uiy = figHeight - (pinPos(i,2) * yScale);  % flip Y

btn = uibutton(fig, 'push', ...

'Text', 'Select', ...

'Position', [uix-20, uiy-15, 40, 30], ...

'ButtonPushedFcn', @(btn, event) moveMotorToPin([pinPos(i,1), pinPos(i,2)], ...

fig, motorPosition), ...

'FontSize', 8);

btn.BackgroundColor = [0.2 0.8 0.2];

end

% If using webcam, you can release it at the end if you want

if exist('cam','var')

clear cam;

end

%% ========================== LOCAL FUNCTIONS ========================== %%

function dialogBox(currentImage)

d = dialog('Position',[300 300 260 150],'Name','Image Acquisition');

textString = "Click Advance when " + currentImage + " is ready";

uicontrol('Parent',d,...

'Style','text',...

'Position',[20 80 220 40],...

'String',textString);

uicontrol('Parent',d,...

'Position',[95 20 70 25],...

'String','Advance',...

'Callback','delete(gcf)');

uiwait(d);

end

function sendRotationsToMotor(targetRot)

assignin('base', 'targetRot', targetRot);

assignin('base', 'newTargetRotAvailable', 1);

set_param('working', 'SimulationCommand', 'update');

fprintf('targetRot = %.3f rev sent to Simulink workspace\n', targetRot);

startSimulinkModel();

end

function slowMoveToRot(targetRot)

try

currentRot = evalin('base','currentRot');

catch

currentRot = 0;

end

maxStep = 0.05;   % rev per step (smaller = slower)

dt      = 0.1;    % seconds between steps

fprintf('Slow move: currentRot = %.3f, targetRot = %.3f\n', currentRot, targetRot);

while abs(targetRot - currentRot) > 1e-3

step = maxStep * sign(targetRot - currentRot);

if abs(targetRot - currentRot) < abs(step)

currentRot = targetRot;

else

currentRot = currentRot + step;

end

assignin('base','currentRot', currentRot);

assignin('base','targetRot', currentRot);

assignin('base','newTargetRotAvailable', 1);

set_param('working', 'SimulationCommand', 'update');

fprintf('  -> commanding %.3f rev\n', currentRot);

pause(dt);

end

fprintf('Reached targetRot = %.3f rev (slow move).\n', targetRot);

end

function startSimulinkModel()

modelName = 'working';

if ~bdIsLoaded(modelName)

try

load_system(modelName);

fprintf('Simulink model "%s" loaded\n', modelName);

catch

fprintf('Could not load Simulink model "%s". Check model name.\n', modelName);

return;

end

end

% If you want auto-start:

% set_param(modelName, 'SimulationCommand', 'start');

% fprintf('Simulation started\n');

end

function targetRot = moveMotorToPin(objectLoc, fig, motorPosition) %#ok<INUSD>

fprintf("Calculating motor rotations for selected pin...\n");

laneCenter_px   = evalin('base', 'laneCenter_px');

inchesPerPixel  = evalin('base', 'inchesPerPixel');

travelPerRev_in = evalin('base', 'travelPerRev_in');

maxRotations    = evalin('base', 'maxRotations');

halfLane_in     = evalin('base', 'halfLane_in');

pinX_px = objectLoc(1);

pinY_px = objectLoc(2); %#ok<NASGU>

fprintf("Pin centroid (pixels): (%.1f, %.1f)\n", pinX_px, pinY_px);

fprintf("Lane center (pixels):  %.1f\n", laneCenter_px);

if isnan(inchesPerPixel)

warning('inchesPerPixel is NaN (no calibration). Rotations will be NaN.');

end

% Pixels -> inches from lane center

offset_in = (pinX_px - laneCenter_px) * inchesPerPixel;  % + right, - left

fprintf("Desired offset from lane center: %.3f in\n", offset_in);

% Clamp to physical lane edges

offset_in_clamped = max(min(offset_in, halfLane_in), -halfLane_in);

if abs(offset_in - offset_in_clamped) > 1e-3

fprintf("Offset clamped from %.3f in to %.3f in (lane edge)\n", ...

offset_in, offset_in_clamped);

end

% Inches -> rotations

targetRot = offset_in_clamped / travelPerRev_in;

% Clamp rotations to mechanical limit

targetRot_clamped = max(min(targetRot, maxRotations), -maxRotations);

if abs(targetRot - targetRot_clamped) > 1e-3

fprintf("Rotations clamped from %.3f rev to %.3f rev (rack limit)\n", ...

targetRot, targetRot_clamped);

end

targetRot = targetRot_clamped;

fprintf("Commanded target rotations from center: %.3f rev\n", targetRot);

% Move gently (slowed) to the new rotation

slowMoveToRot(targetRot);

end